Table 2. Properties of major items/activities included in the inventory

| Item/activity | Description | Reference |
| --- | --- | --- |
| Vermicompost system lifespan | Five years | (Lalander et al., 2015) |
| Cattle manure | 27 kg of manure (wet basis) produced by one cow per day | (MWPS, 1993) |
| Steel parts | Weight of roofing materials and steel columns. | Product catalogue of Roofings (Uganda) Limited. |
| CO2 emissions during manufacture of steel in Uganda | Using scrap metal and electric arc furnace method for manufacturing steel, direct emissions of CO2 (tonnes) = emission factor (1.06) x quantity of steel (tonnes) | (IPCC, 2006a) |
| Energy to produce one tonne of steel in Uganda | Steel is produced mainly from scrap metal using an electric arc furnace and consumes 1860 kWh per tonne of steel produced. | (Komakech et al., 2015a) |
| Concrete mixture | 1:2:4 of cement: sand: gravel by weight. | (FAO, 2011) |
| CO2 emissions in production of cement (tonnes) | Tier one equation used = weight of cement produced x clinker fraction of cement (0.92) x emission factor for clinker (0.52) | (IPCC, 2006a) |
| Energy used in production of one tonne of cement | 105-140 kWh. In this study, 122.5 kWh was used. | (CFA, 2009) |
| Transportation of building materials and equipment/ fertiliser. | 21 tonne UD Nissan dump truck was used to transport building materials. Its emissions (g/h) when empty and when carrying load were assumed to be similar to those of the Volvo A25C dumper truck, which are as follows: Empty 67200 CO2, 65 CO, 38.6 HC and 628 NOx; loaded 85344 CO2, 83 CO, 49 HC and 798 NOx. Transportation of equipment and fertiliser was calculated using the NTM online calculator. | (Andersson, 2005; Lindgren et al., 2002) http://www.ntmcalc.org. |
| Distance to transport building material | Time taken to transport various building materials from their point of origin to the building site in Kampala per truck trip was as follows: 1 hour from cement factory, 1 hour from sand and gravel mining site. |  |
| Direct process emissions from untreated manure per livestock unit (LU) | 2.74 g LU-1day-1 CH4, 4.90 g LU-1day-1 N2O (dumped manure), 1.95 g LU-1day-1 CH4, 0.49 g LU-1day-1 N2O (manure used as fertiliser) NH3 loss 75 g N tonne-1 mainly in the first few days, leaching loss 0.1 g N LU-1day-1 | (IPCC, 2006b) (Sommer and Dahl, 1999) (IPCC, 2006b) |
| Average CO2 emissions from untreated manure | 0.195 kg C tonne-1day-1 | (Sommer and Dahl, 1999) |
| Loss of P from stored manure | 6.3% | (Sommer and Dahl, 1999) |

References

Andersson, M., 2005. Heavy duty vehicles carbon dioxide emissions, a survey on the situation in Sweden. Environmental engineering degree project. Dalarna university college, Sweden.

CFA, 2009. Cement sector program in sub-Saharan Africa. Barriers analysis to CDM., and solutions. Final Report. Carbon Finance Assist (CFA) and the World Bank. Econoler International. 134 pp. http://siteresources.worldbank.org/INTCARFINASS/Resources/FinalReportAfricaCementSector090420.pdf. Accessed on 05.02.2015.

FAO, 2011. Rural structures in the tropics. Design and Development. Food and Agricultural Organization (FAO). Rome. Italy. 461 pp. http://www.fao.org/docrep/015/i2433e/i2433e00.htm. Accessed on 16.04.2015.

IPCC, 2006a. Guidelines for National Greenhouse Gas Inventories. Intergovernmental panel on climate change (IPCC). <http://www.ipcc-nggip.iges.or.jp/public/2006gl/index.htm> accessed 13.06.2013.

Lindgren, M., Pettersson, O., Hansson, P.A., Norén, O., 2002. Engine load pattern and engine exhaust gas emissions from off-road vehicles and methods to reduce fuel-consumption and engine exhaust gas emissions (In Swedish). JTI., - rapport. Lantbruk and Industri. 308 pp.

MWPS, 1993. Handbook, Livestock Waste Facilities. Midwest Plan Service (MWPS), Iowa State University, Ames Iowa 18.